Workflow :

## Phase 1: Initialization

- Script Start: You run demo_feature_extraction.py.

- Entry Point: The script begins execution in the if __name__ == "__main__": block, which immediately calls the main() function.

- Directory Setup: The main() function's first action is os.makedirs('test_results', exist_ok=True). This creates a new folder named test_results in the same directory as the script. exist_ok=True cleverly prevents the script from crashing if the folder already exists from a previous run.

- Signal Generation: The main() function prints "Generating test signals..." and then calls generate_test_signals().

## Phase 2: Synthetic Signal Generation

This phase happens entirely within the generate_test_signals() function.

- Setup: It defines a sampling frequency (fs=1e6 Hz) and duration (duration=0.001 s), which means it will create 1000 data samples for each signal. A time vector t is created to represent these 1000 points.

- Test Signal Creation: It builds four different complex I/Q signals from scratch.

  - QPSK: A digital signal is created by:

    - Choosing random phase "jumps" (0, 90, 180, 270 degrees).

    - Creating complex symbols from these phases (e.g., $1+0j$, $0+1j$, $-1+0j$).

    - Repeating each symbol to simulate a symbol rate (making the signal "blocky").

    - Multiplying the result by a complex carrier wave to shift its frequency to 100 kHz.

    - Adding a small amount of random complex noise.

  - FM: An analog signal is created by:

    - Creating a low-frequency (5 kHz) sine wave (the "modulating" signal).

    - Creating a 150 kHz carrier.

    - The phase of the carrier is modified directly by the modulating sine wave.

    - The final signal is generated as $e^{j \cdot \text{phase}(t)}$, resulting in a signal with constant amplitude but varying frequency.

    - Adding noise.

  - BURST: An on-off signal is created by:

    - Creating a simple 200 kHz carrier wave.

    - Creating an "envelope" array of all zeros.

    - Looping and setting "chunks" of the envelope to 1.0 (e.g., samples 0-200 are 1, 201-499 are 0, 500-700 are 1, etc.).

    - Multiplying the carrier by this 0/1 envelope, effectively turning it on and off.

    - Adding noise.

  - AM: An analog signal is created by:

    - Creating a 120 kHz carrier wave.

    - Creating a 3 kHz modulating sine wave.

    - Creating an amplitude envelope based on the formula $(1 + 0.5 \times \text{modulating\_wave})$.

    - Multiplying the carrier by this analog envelope, resulting in a signal with varying amplitude but constant frequency.

    - Adding noise.

- Return: The function returns a dictionary signals (containing the four numpy arrays), the time vector t, and the sample rate fs back to the main() function.

## Phase 3: Main Processing Loop

Back in main(), the script now loops through the signals dictionary, processing one signal at a time. We will follow the 'qpsk' signal through the entire test_all_features function.

Loop 1: signal_name = 'qpsk'

The main function calls test_all_features('qpsk', iq_signal, t, fs).

Inside test_all_features('qpsk', ...):

- Spectrogram:

  - Calls compute_spectrogram(iq_signal, fs, nperseg=256).

  - Inside compute_spectrogram: This function from signal_features.py uses scipy.signal.spectrogram to perform a Short-Time Fourier Transform (STFT). It breaks the 1000-sample signal into 256-sample segments (with 128 samples of overlap), calculates the Power Spectral Density (PSD) for each, and converts the power to decibels (dB) using $10 \times \log_{10}(\text{power})$.

  - plot_spectrogram_result is called, which uses matplotlib to create a heatmap of the result (time vs. frequency vs. power) and saves it as test_results/qpsk_spectrogram.png.

- Center Frequency & PSD:

  - Calls estimate_center_frequency(iq_signal, fs, method='peak').

  - Inside estimate_center_frequency:

    - Calculates the Fast Fourier Transform (FFT) of the entire signal using fft().

    - Calculates the PSD (power) by taking the squared absolute value of the FFT result.

    - It shifts the FFT output (fftshift) so that 0 Hz is in the center.

    - Because method='peak', it finds the single frequency bin with the most power (np.argmax(psd)) and returns that frequency as the center_freq.

  - plot_psd is called, which plots the PSD (power vs. frequency) and saves it as test_results/qpsk_psd.png.

- Bandwidth:

  - Calls estimate_bandwidth(iq_signal, fs, threshold_db=-10).

  - Inside estimate_bandwidth:

    - It first calls estimate_center_frequency to get the full PSD.

    - It converts the PSD to dB.

    - It finds the maximum power (peak_power_db).

    - It calculates a threshold (e.g., peak_power_db - 10).

    - It finds all frequencies where the power is above this threshold.

    - The bandwidth is calculated as the difference between the highest and lowest frequencies that met this threshold.

- Constellation Diagram:

  - Calls plot_constellation(iq_signal). This is a simple plot. It just creates a scatter plot of the real part (I) vs. the imaginary part (Q) of the complex data.

  - Saves the plot as test_results/qpsk_constellation.png.

- Burst Detection:

  - Calls detect_bursts(iq_signal, fs, threshold_db=-10).

  - Inside detect_bursts:

    - Calculates the signal's instantaneous amplitude, or envelope, using np.abs(iq_data).

    - Converts the envelope to dB.

    - Finds the median power level of the entire signal.

    - Sets a detection threshold 10 dB above this median.

    - Finds all contiguous blocks of samples that are above this threshold.

    - It returns a list of these blocks (bursts) and the envelope itself.

- Duty Cycle:

  - Calls compute_duty_cycle(bursts, len(iq_signal), fs).

  - Inside compute_duty_cycle: It simply sums the total duration (in seconds) of all bursts found in the previous step and divides by the total signal duration (0.001 s).

- Time Domain Plot:

  - Calls plot_time_domain(t, iq_signal, envelope=envelope).

  - This function creates two subplots: one for the I (real) component and one for the Q (imaginary) component over time.

  - It overlays the envelope (from step 5) as a red dashed line.

  - Saves the plot as test_results/qpsk_timedomain.png.

- Modulation Classification:

  - Calls classify_modulation_simple(iq_signal, fs).

  - Inside classify_modulation_simple: This is a heuristic (rule-based) function.

    - It calculates amplitude statistics (how much the amplitude varies).

    - It calculates phase statistics (how much the unwrapped phase varies).

    - It uses a simple if/else tree to make a guess. (e.g., "IF amplitude variation is high AND phase variation is low, THEN classify as 'AM'").

- Round-Trip Test (IQ -> Waveform):

  - Calls iq_to_waveform(iq_signal[:1000], fs).

  - Inside iq_to_waveform: This converts the complex signal back to a real one by interpolating (upsampling) it and shifting its frequency.

- Round-Trip Test (Waveform -> IQ):

  - Calls waveform_to_iq(waveform, fs_real).

  - Inside waveform_to_iq: This uses the Hilbert Transform (scipy.signal.hilbert) to reconstruct the complex analytic signal from the real waveform.

Loop 2, 3, 4:

The script repeats this entire 10-step test_all_features process for the 'fm', 'burst', and 'am' signals, saving their respective plots.

## Phase 4: Summary & Cleanup

- Report Generation: After the main loop is finished, the main() function now has the all_results dictionary, which contains the numerical results for all four signals.

- Formatting: It loops through all_results and builds a large string (summary_text) that neatly formats these results into a table.

- Save & Print: This summary string is:

  - Written to a new file: test_results/summary.txt.

  - Printed to the console.

- Finish: The main() function ends, and the script execution is complete.

Unexpected or Incorrect Results :

The discrepancies stem from specific logic flaws in the signal_features.py module and the parameters used in the demo_feature_extraction.py script.

1. AM Signal Bandwidth is 0.0 kHz

- Unexpected Result: The AM signal shows a bandwidth of 0.0 kHz.

- Expected Result: The AM signal is generated with a 3 kHz modulating tone (fm = 3e3). This should create two sidebands, one at +3 kHz and one at -3 kHz from the carrier. The total bandwidth should be 6.0 kHz.

- Reason: This error is caused by the combination of the estimate_bandwidth function's threshold and the AM signal's modulation index.

  - The estimate_bandwidth function is called with threshold_db=-10. This means it looks for all frequency components that are within -10 dB of the peak power.

  - The AM signal is generated with m = 0.5. The power of the sidebands relative to the carrier (the peak) is calculated as $10 \times \log_{10}((m/2)^2)$.

  - $10 \times \log_{10}((0.5/2)^2) = 10 \times \log_{10}(0.0625) \approx \textbf{-12.04 dB}$.

  - Because the sidebands (-12.04 dB) are weaker than the -10 dB threshold, the only frequency component strong enough to be detected is the carrier itself. This results in the function measuring a bandwidth of 0.

2. QPSK and FM Center Frequencies are Incorrect

- Unexpected Result:

  - QPSK is centered at 88.0 kHz instead of its generated 100 kHz.

  - FM is centered at 130.0 kHz instead of its generated 150 kHz.

- Reason: The demo script uses estimate_center_frequency(..., method='peak').

  - This method finds the single frequency bin with the most power.

  - For modulated signals like QPSK (which has a suppressed carrier) and wideband FM, the carrier frequency is often not the strongest point in the spectrum. The power is distributed among the sidebands.

  - The function is correctly finding the peak sideband (e.g., at 88 kHz) but incorrectly reporting this as the center frequency.

  - Note: The bandwidth calculation is centered correctly (e.g., the FM range is 120-180 kHz, centered at 150 kHz), but the separate "Center Freq" metric reported in the table is wrong because it only used the peak method. Using method='weighted' would likely fix this.

3. BURST Signal Detection is Incorrect

- Unexpected Result: The BURST signal shows 5 bursts and a 47.90% duty cycle.

- Expected Result: The signal is generated with a 200 µs pulse every 500 µs. In a 1000 µs (1 ms) duration, this should create 2 bursts and a 40.0% duty cycle ($2 \times 200 \mu s / 1000 \mu s$).

- Reason: This error is caused by an incorrect parameter in the detect_bursts function call.

  - The function is called with threshold_db=-10.

  - Inside detect_bursts, the threshold is set as threshold = median_power + threshold_db.

  - For the BURST signal, the signal is "off" (just noise) 60% of the time. Therefore, the median_power will be the median power of the noise floor.

  - The code sets the detection threshold to median(noise_floor) - 10. This is 10 dB below the noise floor.

  - As a result, the function isn't detecting the signal bursts. It's detecting random, brief dips in the noise (fades) that momentarily drop below this very low threshold. This results in a random number of "bursts" (in this case, 5).

  - This also explains why QPSK, FM, and AM all show "1 burst" and "100% duty cycle"—their signal power never drops below the noise floor, so they are detected as one continuous burst.

4. FM Modulation Classification is Wrong

- Unexpected Result: The FM signal is classified as "PSK-like" instead of "FM/PM."

- Reason: The heuristic logic in classify_modulation_simple is flawed.

  - The function distinguishes between FM and PSK based on phase_std, which is the standard deviation of np.diff(phase_unwrapped). This measures the standard deviation of the instantaneous frequency.

  - It expects FM to have a high standard deviation (> 1.0) and PSK to have a low one.

  - However, the generated FM signal's frequency varies smoothly (following a sine wave). The PSK signal, in contrast, has an instantaneous frequency of zero except for large, sharp spikes at symbol changes.

  - The FM signal's smooth frequency variation results in a lower standard deviation than the classifier's 1.0 threshold, causing it to be misclassified as "PSK-like."